# WRP Procurement ToolKit – Annex B

Part of Weather Ready Pacific Standard, Optimisation & Maintenance Guidelines Document

Module: ICT & Telemetry Systems

Version: 1.0 | March 2026

When to Use This Module

Use this template when procuring:

- Data management systems

- Communications infrastructure

- Central processing systems

IMPORTANT NOTICE

This Procurement Toolkit is a guidance document only, developed under the Weather Ready Pacific (WRP) Programme to support standardised procurement of Earth observation equipment across Pacific Island Countries and Territories.

It is not a complete tender document. Users may need to adapt all content to:

- National procurement laws and policies

- Local environmental and workplace safety regulations

- Project-specific requirements and site conditions

- Applicable customs, import, and tax requirements

Professional judgment is required. Users should seek appropriate technical and legal advice before finalising any tender.

Limitation of Liability: The WRP Programme, its implementing partners, and supporting organisations accept no liability for any decisions made or actions taken based on this guidance.

Living Document: This toolkit will be updated periodically. Verify you have the latest version through SPREP or the WRP Programme.

Contact: Weather Ready Pacific Programme, SPREP, Apia, Samoa [include the email address of the WRP programme?]

## System Definition

Project Name: _________________________________

System type:

☐Central Data Management System

☐ Telemetry Infrastructure

☐ Data Backup System

☐ Full ICT Upgrade

☐ Partial ICT Upgrade

Number of field stations to be supported: ___________ stations

Existing telemetry: ☐ Cellular network ☐ Satellite network ☐ Mixed ☐ None ☐ Unknown

Current data managements system: ☐ CliDE (database + interface) ☐ Custom database

☐ Excel / manual ☐ None

Current data users: ☐ Forecasters ☐ Hydrologists ☐ Climate scientists ☐ Public website

Required data latency: ☐ Real-time (<5 mins) ☐ Near-real time (<1 hour) ☐ Daily

Number of concurrent users: ___________ internal, ___________ external

Disaster recovery requirements: ☐ Hot standby ☐ Cold standby ☐ Cloud backup only

☐ Unknown

Cybersecurity requirements: ☐ Government network ☐ Public facing ☐ Isolated

☐ Mixed / Unknown

## Tier Selection

| Tier | Typical Use Case | System Type | Recommended For |
| --- | --- | --- | --- |
| 1 (Reference) | National data centre, regional hub, GBON/RBON focal point | Fully redundant servers, multiple communication paths, 24/7 operations | National meteorological service headquarters, regional data centres, GBON data management |
| 2 (Synoptic) | Operational data management, forecasting support | Single server with backup, standard communication links | NMHS central office, backup data centre |
| 3 (Basic) | Limited data management, manual processes | Simple database, minimal automation | Small NMHS, single user, non-critical applications |

Selected Tier: ☐ 1 (Reference) ☐ 2 (Synoptic) ☐ 3 (Basic) ☐ Mixed (specify: _________)

Primary Platform: ☐ On-premises servers ☐ Cloud-based ☐ Hybrid

## System Architecture

This section defines how the system is built, how components interact, and how data flows from sensor to user.

  - Data Acquisition & Processing (Central Systems)

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Data Ingestion Server | Redundant servers (active-active), minimum 16 cores, 64GB RAM, 2TB SSD, 10GbE network | Single server, 8 cores, 32GB RAM, 1TB SSD, 1GbE network | Small server or workstation, 4 cores, 16GB RAM, 500GB HDD |  |
| Database Server | Redundant configuration, PostgreSQL or similar, clustered for high availability, automated failover | Single database server with nightly backups | Lightweight database (SQLite, MySQL) |  |
| Application Server | Load-balanced configuration, supports concurrent users | Single application server | Integrated with database server |  |
| Web Server | Redundant, HTTPS, public facing with CDN optional | Single server, HTTPS | Not required or basic |  |
| Processing Capacity | Real-time data processing, QC, format conversion for all stations (<1 second per station) | Batch processing, near-real-time | Manual processing |  |
| Data Storage | 10-20 TB usable, RAID 10 or equivalent, hot-swappable drives, 5-year data online, 10-year nearline | 2-5 TB, RAID 5 or 6, 3-year data online | 1 TB, basic backup |  |
| Backup System | Automated daily full, hourly incremental, off-site replication (≥100km away), tested monthly | Automated daily full, off-site weekly, tested quarterly | Manual or cloud backup, tested annually |  |
| Disaster Recovery | Hot standby at secondary site, RPO <1 hour, RTO <4 hours | Cold standby, RPO <24 hours, RTO <48 hours | Not required |  |
| Network Architecture | Redundant core switches, firewall, load balancer, VPN for remote access | Single core switch, firewall, VPN | Basic router/firewall |  |
| UPS Power | Online UPS, 4-hour runtime, generator connection, automatic transfer switch | Line-interactive UPS, 2-hour runtime | Basic UPS, 1-hour runtime |  |
| Environment | Dedicated server room, climate-controlled (22°C ±2°C), humidity control, fire suppression, access control | Secure server room or cabinet, climate-controlled | Secure office space |  |

Additional Requirements for Central Systems:

- Operating System: Linux (Ubuntu LTS, Red Hat) or Windows Server (latest stable version)

- Virtualisation: VMware or Hyper-V for Tier 1 (allows flexibility)

- Monitoring: 24/7 system monitoring with alerting for all critical components

- Logging: Centralised logging for security and troubleshooting

- Patch Management: Automated patching schedule with rollback capability

  - ICT Components Complement

| Sensor Type | Tier 1 | Tier 2 | Tier 3 | Included? |
| --- | --- | --- | --- | --- |
| Data Management Software | ✓ Mandatory (full suite) | ✓ Mandatory (standard) | ✓ Mandatory (basic) | ☐ |
| Telemetry Gateway Software | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| Web Portal / API | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| Data Quality Control (QC) | ✓ Mandatory | ✓ Mandatory | ☐ Manual | ☐ |
| Data Visualisation | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| Forecast Integration | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| Metadata Management | ✓ Mandatory | ✓ Mandatory | ☐ Basic | ☐ |
| Alert/Warning System | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| User Management | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| API for External Systems | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| GIS Integration | ☐ Optional | ☐ Optional | ☐ Optional | ☐ |

Selected Sensors for This Procurement:

List the specific sensors required:

| Sensor | Quantity | Manufacturer Preference (if any) | Notes |
| --- | --- | --- | --- |

  - Data Transmission & Telemetry (Central)

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Telemetry Gateway | Multiple gateways for redundancy, supports all field telemetry types (cellular, satellite, radio), store-and-forward buffering | Single gateway, supports primary telemetry types | Simple data import from email/manual |  |
| Communication Protocols | WIS 2.0, MQTT, HTTP/HTTPS, FTP/SFTP, email, multiple simultaneous connections | WIS 2.0, HTTP/HTTPS, FTP | HTTP or manual upload |  |
| Data Format Support | BUFR, CSV, JSON, NetCDF, WaterML2, SeaLevel, TEMP, SYNOP | BUFR, CSV, JSON, TEMP, SYNOP | CSV, manual entry |  |
| Data Ingestion Rate | 1,000+ messages per minute sustained | 100+ messages per minute | 10+ messages per minute |  |
| Latency | <5 seconds from field to database | <1 minute from field to database | <1 hour from field to database |  |
| Network Uplink | Dedicated fiber (minimum 100Mbps), redundant links (different providers), load-balanced | Business broadband (minimum 50Mbps), secondary backup (cellular) | Standard broadband (10Mbps) |  |
| Network Security | Next-generation firewall, IDS/IPS, network segmentation, DMZ for public services, VPN for remote access | Firewall, VPN for remote access | Basic firewall |  |
| Remote Site Monitoring | 24/7 monitoring of all field telemetry links, automated alerting for outages | Daily monitoring, manual alerting | Not required |  |

Telemetry Integration Requirements:

The system must be capable of ingesting data from:

☐ Cellular (4G/LTE) field stations

☐ Iridium satellite field stations

☐ BGAN/Starlink/VSAT field stations

☐ Radio/VHF field stations

☐ Manual data entry (forms, CSV upload)

Data Flow Diagram Requirement regardless of tier:

The bidder must provide a detailed data flow diagram showing:

- End-to-end data flow from field sensors → telemetry → central ingestion → database → applications → users

- Data formats at each stage

- Backup paths and failover mechanisms

- Points of manual intervention

  - Power System (Central)

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Primary Power | Grid connection with dedicated circuit, surge protection at building entry | Grid connection, surge protection | Grid connection |  |
| UPS | Online double-conversion UPS, N+1 redundancy, 4-hour runtime at full load, automatic bypass | Line-interactive UPS, 2-hour runtime | Basic UPS, 1-hour runtime |  |
| Generator | Automatic standby generator, sized for full data centre load, fuel for 72 hours, weekly auto-test | Manual start generator (if available) | Not required |  |
| Power Distribution | Redundant PDUs, circuit breakers, surge protection at rack level | Single PDU, surge protection | Power board with surge protection |  |
| Monitoring | Remote power monitoring: voltage, current, UPS status, generator status, environmental sensors (temp, humidity, water) | Basic power monitoring | Visual check |  |
| Cooling | Precision air conditioning, redundant units, N+1 configuration, temperature set point 22°C ±2°C | Standard air conditioning, single unit | Natural ventilation or standard AC |  |

  - Physical Infrastructure

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Server Room | Dedicated room, raised floor (optional), fire suppression (FM200 or equivalent), biometric access control, CCTV | Dedicated room or secured cabinet, access control | Secure office space |  |
| Racks | Standard 19-inch server racks, seismic-rated, cable management, lockable doors | Standard racks or cabinets, lockable | Desktop or small cabinet |  |
| Cabling | Structured cabling (Cat6a or Cat7), fibre optic for backbone, labelled, documented, cable trays | Structured cabling, labelled, documented | Basic cabling |  |
| Environmental Monitoring | Temperature, humidity, water leak detection, smoke detection, remote alerts | Temperature monitoring | Not required |  |
| Lightning Protection | Full building lightning protection, surge arrestors at all external cable entries | Surge arrestors at building entry | Basic surge protection |  |
| Grounding | Dedicated telecommunications ground, <5 ohm, separate from building ground where possible | Combined ground, <10 ohm | Basic ground |  |
| Access Security | 24/7 security (or building security), access logs, visitor procedures | Locked access, after-hours building security | Locked office |  |

  - Software & Data Management

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Data Management Platform | CliDE (preferred) or equivalent full-featured CDMS. CliDE serves as both the database (PostgreSQL-based storage) and the interface for data entry, QC, and retrieval. Configured for all station types, automated data flows. | CliDE (preferred) or equivalent with core features. Complete CliDE system including database and interface for primary station types. | CliDE basic installation or spreadsheet-based management. Core database functionality with limited interface. |  |
| Database Component | CliDE PostgreSQL database (or equivalent), fully indexed, optimised for time-series climate data, with PostGIS for spatial capabilities. Supports all WMO and regional data formats. | CliDE PostgreSQL database (or equivalent) with core schema, supports primary station types. | CliDE database (or equivalent) with basic schema. |  |
| Interface Component | CliDE application interface (or equivalent) with full functionality: data entry, quality control, visualisation, reporting, user administration, metadata management, export capabilities. | CliDE application interface (or equivalent) with core functionality for data entry, QC, and reporting. | CliDE application interface (or equivalent) with basic data entry and retrieval. |  |
| Data Quality Control | CliDE QC module (or equivalent): automated QC (range, rate-of-change, persistence, spatial consistency), configurable thresholds, manual override, QC flags stored in database with audit trail. | CliDE QC module (or equivalent): automated QC (range, rate-of-change), configurable thresholds. | Manual QC using CliDE interface. |  |
| Metadata Management | CliDE metadata module (or equivalent): Full WIGOS metadata integration, OSCAR/Surface export, automated metadata updates, version history stored in database. | CliDE metadata module: WIGOS metadata capture, manual export. | Basic station metadata in CliDE. |  |
| Data Archiving | CliDE archiving module (or equivalent): Automated archiving to nearline/cloud, long-term preservation, data versioning within CliDE database structure. | Manual archiving using CliDE export functions. | Backup only. |  |
| Data Access & Visualisation | CliDE visualisation module (or equivalent) plus web-based portal: maps, graphs, tables, export functions, user-configurable dashboards. CliDE interface for advanced users. | CliDE interface with basic maps and graphs. Web portal optional. | CliDE basic interface. |  |
| Forecast Integration | Real-time data feed from CliDE to forecast models and workstations, BUFR/SYNOP generation for GTS/WIS 2.0 via CliDE export functions. | Data feed from CliDE to forecast workstations, basic BUFR generation. | Manual data export from CliDE. |  |
| Alert/Warning System | Multi-channel alerts (SMS, email, dashboard) based on CliDE QC flags and thresholds, configurable thresholds, escalation procedures. | Basic email alerts from CliDE threshold checks. | Manual monitoring via CliDE. |  |
| User Management | CliDE user administration (or equivalent): Role-based access control, audit logs, group permissions, 2FA recommended. | Basic CliDE user accounts, password access. | Single user in CliDE. |  |
| API | RESTful API for external systems, documented, versioned, rate limited. Direct database access not permitted; all access via API or CliDE interface. | Basic API or file export from CliDE. | Manual file transfer. |  |

  - Data Management Requirements

| Requirement | Specification | Compliant? |
| --- | --- | --- |
| WIS 2.0 Compliance | System must generate BUFR messages and push to WIS 2.0 global/regional nodes |  |
| GBON/RBON Reporting | Automated reporting of station status, data availability, metadata to WMO systems |  |
| Data Sovereignty | All data stored on NMHS-controlled infrastructure (on-premises or designated cloud with sovereignty clauses) |  |
| Open Standards | Data accessible in open formats (CSV, NetCDF, BUFR), no proprietary lock-in |  |
| Long-term Preservation | Data retention policy: 5 years online, 10 years nearline, indefinite archive (Tier 1) |  |

  - Integration with Existing Systems

The proposed system must integrate with the following systems:

| System | Integration Requirement | Compliant? |
| --- | --- | --- |
| [e.g., Existing CliDE Installation] | Data migration from existing CliDE database to new CliDE installation, preserving all historical data, QC flags, metadata, and audit logs. Seamless transition for users. |  |
| [e.g., Forecaster Workstation] | Real-time data feed from CliDE database in required format. |  |
| [e.g., WIS 2.0 Global Node] | Data push in BUFR format from CliDE. |  |
| [e.g., Regional Data Centres] | Data sharing agreements, automated exports from CliDE. |  |
| [e.g., Public Website] | Automated data feed from CliDE database for public display. |  |
| [e.g., Early Warning System] | Real-time data feed from CliDE with alerts based on QC flags. |  |

Integration Testing:

The contractor shall demonstrate successful integration during:

- Factory Acceptance Test (simulated environment)

- Site Acceptance Test (live with field stations)

- Data migration validation

- Performance testing under load

## Technical Specification by Tier

  - Hardware Specifications

Tier 1 - Reference Data Centre

| Component | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Database Server | Dual CPU (≥16 cores total), 128GB RAM, 2TB NVMe SSD (RAID 1) for OS/logs, 10TB SAS SSD (RAID 10) for data, dual 10GbE |  |  |
| Application Server | 16 cores, 64GB RAM, 1TB SSD, dual 10GbE, load-balanced configuration (2 units) |  |  |
| Web Server | 8 cores, 32GB RAM, 500GB SSD, dual 10GbE, load-balanced (2 units) |  |  |
| Storage System | SAN or NAS with 20-50TB usable, RAID 10, redundant controllers, redundant power, 10GbE connectivity |  |  |
| Backup System | 50-100TB capacity, tape or cloud replication, automated backup software |  |  |
| Network Core | Redundant core switches, 48 ports 10GbE, stacking capability, redundant power |  |  |
| Firewall | Next-generation firewall, throughput ≥1Gbps, IPS/IDS, VPN capability |  |  |
| UPS | 20-30kVA online UPS, 4-hour runtime, N+1 configuration |  |  |
| Generator | 50-100kVA automatic standby, 72-hour fuel capacity, weekly auto-test |  |  |
| Cooling | 10-15kW precision cooling, redundant units, N+1 configuration |  |  |

Tier 2 - Synoptic Data Centre

| Component | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Server | Single server: 16 cores, 64GB RAM, 2TB SSD (RAID 10), 1GbE network |  |  |
| Storage | 5-10TB usable, RAID 5 or 6, internal or external storage |  |  |
| Backup | External hard drives or cloud backup, automated daily backup |  |  |
| Network | Core switch, 24 ports 1GbE, firewall with VPN |  |  |
| UPS | 5-10kVA line-interactive UPS, 2-hour runtime |  |  |
| Cooling | Standard air conditioning |  |  |

Tier 3 - Basic System

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Workstation | High-end workstation or small server: 8 cores, 32GB RAM, 1TB storage |  |  |
| Backup | External hard drive, manual or automated backup |  |  |
| Network | Basic router/firewall |  |  |
| UPS | Desktop UPS, 1-hour runtime |  |  |

  - Software Specifications

| Parameter | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- | --- | --- |
| Operating System | Ubuntu or Windows | Ubuntu or Windows | Ubuntu or Windows |  |  |
| Database Platform | PostgreSQL 14+ with PostGIS | PostgreSQL 14+ or MySQL | PostgreSQL, MySQL, or SQLite |  |  |
| CliDE - Database Component | Full CliDE PostgreSQL database: climate data schema, WIGOS metadata, QC flags, audit logs, PostGIS, full indexing | Core CliDE database: primary tables, basic metadata, QC flags, standard indexing | Basic CliDE schema: core surface data, essential metadata |  |  |
| CliDE - Application Interface | Full CliDE suite: data entry, QC (automated + visual), advanced retrieval/visualisation, metadata management, reporting, user administration, system administration | Core CliDE suite: data entry, QC (automated), standard retrieval, basic metadata, user accounts, standard reports | Basic CliDE interface: simple data entry, basic viewing, limited export |  |  |
| Web Interface | Real-time dashboard, interactive maps, configurable views, mobile-responsive | Basic data display, pre-configured views | Not required |  |  |
| API Access | RESTful API with CRUD, authentication, rate limiting, full documentation | Basic read-only API with authentication | Manual file export |  |  |
| Data Format Support | BUFR, CSV, JSON, NetCDF, WaterML2, SeaLevel, TEMP, SYNOP | BUFR, CSV, JSON, TEMP, SYNOP | CSV |  |  |
| WIS 2.0 Compliance | BUFR generation, automated push to global/regional nodes | BUFR generation, manual or automated push | Not required |  |  |
| GBON/RBON Reporting | Automated station status and data availability reporting | Manual reporting | Not required |  |  |
| Backup Software | Enterprise: online backup, deduplication, encryption, off-site replication, point-in-time recovery | Standard: daily backup, off-site weekly | Built-in OS backup |  |  |
| Monitoring Software | Comprehensive: availability, performance, storage, backup monitoring; SMS/email alerts | Basic: uptime, disk space, backup monitoring; email alerts | Manual monitoring |  |  |
| Security Software | Antivirus, IDS/IPS, log monitoring, vulnerability scanning | Antivirus, host firewall | Antivirus |  |  |
| Authentication | Centralised (LDAP/AD), MFA for privileged users | Local authentication, password policy | Local authentication |  |  |
| Audit Logging | Comprehensive logs (login, data changes, QC, exports); 12-month retention | Basic logs (login, data changes); 6-month retention | Basic or none |  |  |
| Documentation | Full suite: installation, admin, user (by role), API, database schema, DR procedures | Core: installation, admin, user, troubleshooting | Basic user manual |  |  |
| Software Updates | 5 years free updates | 3 years free updates | 1-year free updates |  |  |
| License Model | Perpetual license or annual subscription | Perpetual license or annual subscription | Perpetual or included |  |  |

## Spare Parts Requirements by Tier

#### Tier 1 Spare Parts Kit

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Servers | Database server (complete, configured) | 1 |  |  |
|  | Application server (complete) | 1 |  |  |
|  | Web server (complete) | 1 |  |  |
| Storage | Spare HDD/SSD (same as installed) | 4 |  |  |
|  | Spare RAID controller | 1 |  |  |
| Network | Core switch | 1 |  |  |
|  | Firewall | 1 |  |  |
|  | Access switch | 1 |  |  |
| Power | UPS batteries (full set) | 2 sets |  |  |
|  | PDU | 1 |  |  |
| Cables | Fibre optic patch cables (various lengths) | 10 |  |  |
|  | Ethernet cables (various lengths) | 20 |  |  |
| Miscellaneous | Console cable, USB drives for recovery | 1 kit |  |  |
| TOTAL (Tier 1) |  |  |  |  |

#### Tier 2 Spare Parts Kit

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Servers | Server (complete) | 1 |  |  |
| Storage | Spare HDD/SSD | 2 |  |  |
| Network | Spare switch | 1 |  |  |
| Power | UPS batteries | 1 set |  |  |
| TOTAL (Tier 2) |  |  |  |  |

#### Tier 3 Spare Parts Kit

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Workstation | Complete workstation | 1 |  |  |
| Power | UPS | 1 |  |  |
| TOTAL (Tier 3) |  |  |  |  |

## Maintenance Schedule

Tier 1 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Daily | System health checks, review monitoring alerts, check backup status, review security logs | 1 hour | System Administrator |
| Weekly | Performance review, disk space monitoring, review failed jobs, test alerting | 2 hours | System Administrator |
| Monthly | Apply security patches, review user accounts, audit logs, test backup restore (sample), UPS battery test | 4 hours | System Administrator |
| Quarterly | Full backup restore test, disaster recovery drill, security vulnerability scan, generator test (if installed) | 1 day | System Administrator + Support |
| Annually | Hardware inspection, cooling system check, power system load test, update disaster recovery plan, capacity planning | 2 days | System Administrator + Specialist |
| As Required | Hardware replacement (failed components), software upgrades, security incident response | Variable | System Administrator |

Tier 2 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Daily | Quick system health check | 30 min | System Administrator |
| Weekly | Backup verification | 1 hour | System Administrator |
| Monthly | Security patches, disk space review | 2 hours | System Administrator |
| Quarterly | Test backup restore | 4 hours | System Administrator |
| Annually | Full system review, capacity planning | 1 day | System Administrator |

Tier 3 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Weekly | Check system health | 30 min | Technician |
| Monthly | Verify backups | 1 hour | Technician |
| Quarterly | Update software | 2 hours | Technician |

## Field Verification & Calibration

  - System Testing Requirements

| Item | Tier 1 | Tier 2 | Tier 3 | Included? |
| --- | --- | --- | --- | --- |
| Unit Testing | All components tested individually | All components tested | Basic testing | ☐ |
| Integration Testing | Full system integration with field stations | Primary integration testing | Basic connectivity | ☐ |
| Performance Testing | Load testing at 2x expected data volume | Load testing at expected volume | Not required | ☐ |
| Failover Testing | Simulated failure of all redundant components | Basic failover testing | Not required | ☐ |
| Disaster Recovery Testing | Full DR drill, data restoration from backup | Backup restore test | Manual restore test | ☐ |
| Security Testing | Vulnerability scan, penetration test | Vulnerability scan | Basic security check | ☐ |
| Acceptance Testing | 30-day continuous operation, 99.5% availability | 30-day operation, 99% availability | 30-day operation | ☐ |

  - Data Migration Testing (if replacing existing systems)

| Test Type | Description | Pass/Fail Criteria |
| --- | --- | --- |
| Data Completeness | All historical data migrated successfully | 100% of records accounted for |
| Data Integrity | No corruption, all relationships intact | Random sample verification |
| Metadata Migration | All station metadata, QC flags, audit logs migrated | Complete and accurate |
| User Acceptance | End users can access and work with migrated data | All critical workflows functional |

## Warranty & Services

  - Warranty Requirements

| Component | Tier 1 | Tier 2 | Tier 3 | Bidder Offer |
| --- | --- | --- | --- | --- |
| Hardware Warranty | Minimum 5 years, next-business-day on-site for servers, 3 years for other equipment | Minimum 3 years, next-business-day on-site for servers | Minimum 1 year, return-to-base |  |
| Software Warranty | 12 months, includes updates and bug fixes | 12 months, includes updates | 12 months |  |
| Response Time | <4 hours for critical failure acknowledgment, <24 hours for resolution (hardware) | <24 hours acknowledgment, <3 days resolution | <5 days |  |
| On-site Support | Included for critical failures | On-site where available | Not included |  |
| Parts Replacement | Advance replacement within 24 hours | Advance replacement within 1 week | Replacement within 1 month |  |

  - Service Level Agreement (SLA) Requirements

The contractor shall provide a detailed SLA including:

| SLA Element | Tier 1 Requirement | Tier 2 Requirement | Bidder Offer |
| --- | --- | --- | --- |
| System Availability | 99.9% (excluding planned maintenance) | 99.5% |  |
| Planned Maintenance | Max 4 hours/month, after-hours, 2 weeks’ notice | Max 8 hours/month, 1 week notice |  |
| Helpdesk Hours | 24/7/365 | Business hours (8am-5pm local) with emergency after-hours |  |
| Critical Failure Response | <1 hour acknowledgment, <4 hours remote, <24 hours on-site | <4 hours acknowledgment, <24 hours remote |  |
| Security Patch Application | Within 7 days of release (critical: 24 hours) | Within 30 days |  |
| System Availability | 99.9% (excluding planned maintenance) | 99.5% |  |
| Planned Maintenance | Max 4 hours/month, after-hours, 2 weeks’ notice | Max 8 hours/month, 1 week notice |  |

  - Post-Warranty Support Options

The bidder must provide pricing for post-warranty support:

| Option | Description | Annual Cost | Notes |
| --- | --- | --- | --- |
| Option 1: Comprehensive | Full hardware and software support, 24/7 helpdesk |  |  |
| Option 2: Hardware Only | Hardware replacement only |  |  |
| Option 3: Software Only | Software updates and support |  |  |
| Option 4: Time & Materials | Support charged as needed |  |  |
| Option 5: Training & Consulting | Staff training, system optimisation |  |  |

## Training Requirements

| Training Type | Audience | Duration | When | Must Include |
| --- | --- | --- | --- | --- |
| System Administrator Training | ICT staff, system administrators | 5 days | During installation | Installation, configuration, maintenance, backup/restore, security, troubleshooting, performance tuning |
| Database Administrator Training | Data managers, ICT staff | 3-5 days | During installation | Database configuration, backups, queries, optimisation, metadata management |
| User Training | Forecasters, hydrologists, technicians | 2-3 days | After installation | Data access, visualisation, export, QC interpretation, dashboards |
| Security Training | All users with system access | 1 day | After installation | Security policies, password management, phishing awareness, incident reporting |
| Disaster Recovery Training | System administrators | 1 day | After installation | DR procedures, failover, restore from backup, testing |
| Train-the-Trainer | Senior ICT staff | 2 days | After installation | How to train new staff, maintain documentation |

Training materials required:

☐ System administrator manuals (printed + digital) in English

☐ User manuals (printed + digital) in English

☐ Database administrator manuals (printed + digital) in English

☐ Security policy document

☐ Disaster recovery procedure manuals (printed + digital) in English

☐ Quick reference guide for common tasks

☐ Video tutorials (recommended)

☐ Troubleshooting guide and flowchart

☐ Preventative maintenance checklist

☐ API documentation (printed + digital) in English

Training Venue:

- First session (operator): At NMHS or online

- Second session (maintenance/installation): At NMHS, including field installation at 1-2 sites

- Software training: At NMHS computer lab

Training Evaluation:

- Pre- and post-training assessments required

- Practical demonstration of competency (each participant must perform tasks)

- Certificate of completion for each participant

- Training report with recommendations for additional support

## Technical Documentation

The contractor must provide comprehensive technical documentation:

| Document | Description | Format | Quantity |
| --- | --- | --- | --- |
| System Architecture Document | Overall architecture, component diagrams, data flows, network diagrams | Digital + 2 hard copies | 2 sets |
| Installation & Configuration Guide | Step-by-step installation, configuration settings, network setup | Digital + 2 hard copies | 2 sets |
| System Administration Manual | Daily operations, monitoring, backups, user management, security, troubleshooting | Digital + 2 hard copies | 2 sets |
| User Manual | End-user guide: accessing data, visualisation, export, dashboards | Digital + 2 hard copies | 5 sets |
| Database Schema | Complete database schema documentation, relationships, tables, views | Digital | 2 sets |
| API Documentation | Complete API specification, endpoints, authentication, examples | Digital | 2 sets |
| Security Policy | Access controls, password policy, audit logging, incident response | Digital + 2 hard copies | 2 sets |
| Disaster Recovery Plan | Step-by-step recovery procedures, contact lists, restoration testing schedule | Digital + 2 hard copies | 2 sets |
| Backup & Restore Procedures | Detailed procedures for all backup types, restore testing, off-site storage | Digital + 2 hard copies | 2 sets |
| As-Built Documentation | Final configuration, IP addresses, network diagrams, user accounts, system settings | Digital + 2 hard copies | 2 sets |
| System Architecture Document | Overall architecture, component diagrams, data flows, network diagrams | Digital + 2 hard copies | 2 sets |

  - Documentation Quality Requirements

- All documentation must be in English

- Language must be clear, avoiding unnecessary jargon

- Diagrams and illustrations must be used extensively

- Troubleshooting guides must include decision trees

- All documents must include revision control and date

- Digital copies must be searchable PDFs

## Lifecycle Cost Calculation

| Cost Category |  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 | 5-Year Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Capital costs | Server hardware |  |  |  |  |  |  |
|  | Storage system |  |  |  |  |  |  |
|  | Network equipment |  |  |  |  |  |  |
|  | UPS / power |  |  |  |  |  |  |
|  | Cooling (if new) |  |  |  |  |  |  |
|  | Software licenses (one-time) |  |  |  |  |  |  |
|  | Installation |  |  |  |  |  |  |
|  | Training |  |  |  |  |  |  |
| Recurring costs | Software maintenance (annual) |  |  |  |  |  |  |
|  | Cloud services (if applicable) |  |  |  |  |  |  |
|  | Internet/telecom links |  |  |  |  |  |  |
|  | Electricity |  |  |  |  |  |  |
| Maintenance | Hardware support contract |  |  |  |  |  |  |
|  | Spare parts |  |  |  |  |  |  |
|  | Consumables (tapes, disks) |  |  |  |  |  |  |
|  | Staff time (estimated) |  |  |  |  |  |  |
|  | External support |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |

Note: ICT systems typically have a 5-year hardware refresh cycle. Plan for replacement or major upgrade in Year 5-6, if applicable.

Further notes:

- Include all costs in [USD or local currency]

- Include inflation if multi-year contract

- Separate optional items clearly

## Pacific Requirements

All land-based monitoring systems must meet these additional requirements:

| Requirement | Specification | Compliant? |
| --- | --- | --- |
| Power Stability | Pacific grid power can be unreliable; UPS and generator (Tier 1) essential |  |
| Temperature | Server rooms must maintain 22°C ±2°C; Pacific ambient temperatures require reliable cooling |  |
| Humidity | High humidity can damage equipment; humidity control required in server rooms |  |
| Cyclone Preparedness | Critical systems must have documented procedures for cyclone events (shutdown, backup, etc.) |  |
| Remote Support | Internet outages common; systems must support local fallback operations |  |
| Limited Local Support | Equipment must be supportable by local ICT staff with remote vendor assistance |  |
| Data Sovereignty | Cloud solutions must ensure data stored within Pacific or with sovereignty guarantees |  |
| Customs / Import | Hardware imports require customs clearance; plan for delays |  |
| Limited Bandwidth | Some sites have limited bandwidth; data transfer must be optimised |  |
| Multi-lingual Support | Consider local language support for user interfaces |  |
| Training Continuity | Staff turnover is high; documentation must be comprehensive for self-training |  |
| Spare Parts Availability | Critical spares must be held locally; shipping can take weeks |  |

## Compliance Checklist

Before finalising your tender, verify:

| Check | Completed? |
| --- | --- |
| Tier selection matches operational needs | ☐ |
| System architecture fully defined | ☐ |
| Data volume and station count confirmed | ☐ |
| Hardware specifications appropriate for data volume | ☐ |
| Redundancy requirements defined | ☐ |
| Backup strategy defined (frequency, retention, off-site) | ☐ |
| Disaster recovery requirements specified | ☐ |
| Telemetry integration requirements defined | ☐ |
| Network security requirements specified | ☐ |
| Power system sized for central facility (UPS, generator) | ☐ |
| Cooling requirements specified for server room | ☐ |
| Physical security (access control, fire suppression) defined | ☐ |
| Software requirements (data management, QC, visualisation) specified | ☐ |
| Integration with existing systems defined | ☐ |
| Data migration plan (if applicable) | ☐ |
| WIS 2.0 and GBON/RBON compliance verified | ☐ |
| Data sovereignty requirements addressed | ☐ |
| Spare parts kit matches tier | ☐ |
| Maintenance schedule defined | ☐ |
| Testing requirements (performance, security, DR) specified | ☐ |
| Warranty and SLA requirements specified | ☐ |
| Training requirements fully defined | ☐ |
| Technical documentation requirements included | ☐ |
| Pacific requirements all included (Section 12) | ☐ |
| Lifecycle cost worksheet completed (5-year TCO) | ☐ |
| Budget confirmed for capital AND 5-year operation (including support contracts) | ☐ |
| Site survey completed for server room (power, cooling, space) | ☐ |

## Additional Information for Bidders

  - Existing Infrastructure Information

| Item | Details |
| --- | --- |
| Current data management system |  |
| Current database platform |  |
| Current data volume (GB/year) |  |
| Number of field stations |  |
| Existing telemetry systems |  |
| Existing network connectivity |  |
| Server room location (if any) |  |
| Power capacity available |  |
| Cooling capacity available |  |
| Space available (racks) |  |

  - Network Connectivity

| Site | Primary Link | Backup Link | Bandwidth | Provider |
| --- | --- | --- | --- | --- |
| HQ Office |  |  |  |  |
| Data Centre |  |  |  |  |

  - Special Local Requirements

Import duties: ☐ NMHS responsible ☐ Bidder responsible

Customs clearance: ☐ NMHS assists ☐ Bidder responsible

Data sovereignty laws: ☐ Yes (specify: ___________) ☐ No ☐ Unknown

Government network security requirements: ☐ Yes (specify: ___________) ☐ No ☐ Unknown

Local taxes: ☐ Applicable ☐ Exempt (specify)

Work permits: ☐ Required ☐ Not required

Site access restrictions: _________________________________