# WRP Procurement ToolKit – Annex B

Part of Weather Ready Pacific Standard, Optimisation & Maintenance Guidelines Document

Module: Marine Observation Systems

Version: 1.0 | March 2026

When to Use This Module

Use this template when procuring:

- Tide gauges

- Wave buoys

- Sea surface temperature sensors

- Coastal weather stations

- Tsunami warning stations

- Oceanographic mooring stations

IMPORTANT NOTICE

This Procurement Toolkit is a guidance document only, developed under the Weather Ready Pacific (WRP) Programme to support standardised procurement of Earth observation equipment across Pacific Island Countries and Territories.

It is not a complete tender document. Users may need to adapt all content to:

- National procurement laws and policies

- Local environmental and workplace safety regulations

- Project-specific requirements and site conditions

- Applicable customs, import, and tax requirements

Professional judgment is required. Users should seek appropriate technical and legal advice before finalising any tender.

Limitation of Liability: The WRP Programme, its implementing partners, and supporting organisations accept no liability for any decisions made or actions taken based on this guidance.

Living Document: This toolkit will be updated periodically. Verify you have the latest version through SPREP or the WRP Programme.

Contact: Weather Ready Pacific Programme, SPREP, Apia, Samoa [include the email address of the WRP programme?]

## System Definition

Project Name: _________________________________

Number of station(s): _____

Station types:

☐ Tide gauge (coastal sea level) _____ stations

☐ Wave buoy (offshore) _____ stations

☐ Sea Surface Temperature (SST) sensor _____ stations

☐ Coastal weather station (marine) _____ stations

☐ Tsunami warning station _____ stations

☐ Oceanographic mooring _____ stations

Location(s): _________________________________

Water depth at deployment: _____ m (for buoys/moorings)

Primary purpose: ☐ Tsunami warning ☐ Storm surge monitoring ☐ Marine forecasting

☐ Climate monitoring ☐ Port operations ☐ Research

Power available at site: ☐ Grid ☐ Solar only ☐ Mains powers (coastal sites only)

☐ No power available ☐ Wave/solar hybrid ☐ Unknown

Cellular coverage: ☐ Yes ☐ No ☐ Partial ☐ Unknown

Vandalism risk at site: ☐ Low ☐ Medium ☐ High (navigation lights, anti-theft measures required) ☐ Unknown

Site access: ☐ Road ☐ Boat ☐ Walking only

GLOSS Compliance: ☐ Core network ☐ Supplementary ☐ Not applicable

## Tier Selection

| Tier | Typical Use Case | International Standard | Recommended For |
| --- | --- | --- | --- |
| 1 (Reference) | Climate monitoring, sea level rise, tsunami warning, research | GLOSS, GCOS | Capital ports, climate reference sites, tsunami warning network, research stations |
| 2 (Synoptic) | Operational marine forecasting, storm surge, port operations | WMO Class 2 | Secondary ports, coastal communities, marine forecast areas |
| 3 (Basic) | Community-based monitoring, recreational, education | Basic standard | Remote islands, community early warning, schools, aquaculture |

Selected Tier: ☐ 1 (Reference) ☐ 2 (Synoptic) ☐ 3 (Basic) ☐ Mixed (specify: _________)

Note: If a single site will contain equipment from different tiers (e.g., a Tier 1 wind sensor with a Tier 3 rain gauge), this must be clearly specified. The procurement documents should list the required tier for each sensor or system component.

** Your selected tier will dictate the rest of the tender

## System Architecture

This section defines how the system is built, how components interact, and how data flows from sensor to user.

  - Data Acquisition & Processing (Datalogger)

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Processor | 32-bit, industrial grade, low power consumption, -40°C to +70°C operating range | 16-bit or 32-bit, industrial grade, -30°C to +60°C | 16-bit, commercial grade, 0°C to +50°C |  |
| Memory | ≥256 MB, ≥1 year data storage at 1-minute intervals (high-frequency wave data requires more) | ≥64 MB, ≥6 months data storage | ≥16 MB, ≥3 months data storage |  |
| Analog Channels | ≥8 differential or 16 single-ended | ≥4 differential or 8 single-ended | ≥2 differential or 4 single-ended |  |
| Digital Channels | ≥4 digital I/O, ≥2 serial/RS232, SDI-12 (≥5 sensors) | ≥2 digital I/O, ≥1 serial, SDI-12 capable | SDI-12 capable |  |
| Pulse Counters | ≥2 high-speed pulse counters | ≥2 pulse counters | ≥1 pulse counter |  |
| Scan Rate | ≤1 second for critical sensors (wave bursts: 1-2 Hz minimum) | ≤5 seconds | ≤15 seconds |  |
| Wave Burst Sampling | Configurable burst sampling: 2048 samples at 1-2 Hz, programmable interval | Basic burst capability | Not required |  |
| A/D Resolution | 24-bit | 16-bit | 16-bit |  |
| Accuracy | ±0.02% of reading | ±0.1% of reading | ±0.25% of reading |  |
| Time Synchronization | GPS-integrated, automatic daily sync, <1ms accuracy (critical for tsunami detection) | GPS or NTP, <1 second accuracy | Manual or NTP when available |  |
| Real-Time Clock | Battery-backed, <2 minutes/year drift | Battery-backed, <5 minutes/year drift | Battery-backed, <10 minutes/year drift |  |
| Communications Ports | Ethernet, USB, RS-232, RS-485, maintenance port, Iridium modem interface | USB, RS-232, maintenance port | USB or serial, maintenance port |  |
| Data Formats | Native: binary; Export: CSV, JSON, NetCDF, SeaLevel format | Export: CSV, JSON | Export: CSV |  |
| Remote Programming | Full remote configuration and firmware update | Remote configuration | Manual only |  |
| Diagnostics | Comprehensive self-diagnostics, watchdog timer, voltage monitoring, sensor health checks, leak detection | Basic self-diagnostics, watchdog timer | Watchdog timer |  |

Additional Datalogger Requirements for Marine Systems:

- Low Power Design: Must consume <1mA in sleep mode for buoy applications

- Leak Detection: Built-in moisture/humidity sensor with alarm

- Wave Processing: Onboard calculation of wave statistics (Hs, Tz, Tp, direction)

- Tsunami Detection: Real-time processing for rapid sea level changes (Tier 1)

- Data Compression: Efficient compression for satellite transmission

- Maintenance Port: Waterproof connector for local laptop connection

- Enclosure: Marine-grade, waterproof (IP68 for submersible applications, IP66 for above-deck)

  - Sensor Complement

| Sensor Type | Tier 1 | Tier 2 | Tier 3 | Included? |
| --- | --- | --- | --- | --- |
| Sea Level / Tide | ✓ Mandatory | ✓ Mandatory | ✓ Mandatory | ☐ |
| Sea Surface Temperature (SST) | ✓ Mandatory | ☐ Optional | ☐ Optional | ☐ |
| Wave Height | ✓ Mandatory | ☐ Optional | ☐ Optional | ☐ |
| Wave Period | ✓ Mandatory | ☐ Optional | ☐ Optional | ☐ |
| Wave Direction | ✓ Mandatory | ☐ Optional | ☐ Optional | ☐ |
| Barometric Pressure | ✓ Mandatory | ✓ Mandatory | ☐ Optional | ☐ |
| Air Temperature | ✓ Mandatory | ☐ Optional | ☐ Optional | ☐ |
| Wind Speed/Direction | ☐ Optional | ☐ Optional | ☐ Optional | ☐ |
| Conductivity/Salinity | ☐ Optional | ☐ Optional | ☐ Optional | ☐ |
| Dissolved Oxygen | ☐ Optional | ☐ Optional | ☐ Optional | ☐ |
| Turbidity | ☐ Optional | ☐ Optional | ☐ Optional |  |
| pH | ☐ Optional | ☐ Optional | ☐ Optional |  |
| Currents (ADCP) | ☐ Optional | ☐ Optional | ☐ Optional |  |

Selected Sensors for This Procurement:

List the specific sensors required:

| Sensor | Quantity | Manufacturer Preference (if any) | Notes (e.g., depth range, expected max wave height) |
| --- | --- | --- | --- |

  - Sea Level Sensor Type Selection LEFT OFF

| Sensor Type | Advantages | Disadvantages | Suitable for Tier | Selected? |
| --- | --- | --- | --- | --- |
| Radar (Microwave) | Non-contact, no moving parts, low maintenance, good for storm surge | Affected by extreme wave overtopping, requires stable mounting | 1, 2 | ☐ |
| Pressure (Bottom-mounted) | High accuracy, protected from waves, good for tsunami | Requires diving for installation, biofouling, drift over time | 1, 2 | ☐ |
| Bubbler (Pneumatic) | No electronics in water, reliable, good for silty environments | Compressed gas required, maintenance, slower response | 1, 2 | ☐ |
| Float & Shaft Encoder | Simple, reliable, direct mechanical measurement | Moving parts, requires stilling well, maintenance | 2, 3 | ☐ |
| Acoustic | Non-contact, moderate cost | Affected by temperature gradients, less accurate in rough seas | 2, 3 | ☐ |
| Staff Gauge (Manual) | Zero cost, no power required | Manual reading only, no telemetry | All (as backup) | ☐ |

Selected Sensor Type: _________________________________

Justification: ________________________________

  - Data Transmission & Telemetry

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Primary Communication | Dedicated: ☐ Cellular (4G/LTE) ☐ Satellite (BGAN/Starlink/VSAT) ☐ Radio | ☐ Cellular ☐ Satellite ☐ Radio (choose one) | ☐ Cellular ☐ Satellite ☐ Manual download |  |
| Secondary Communication | ☐ Cellular backup (if primary satellite) ☐ Satellite backup (if primary cellular) ☐ Iridium SBD | ☐ Iridium SBD where critical ☐ Not required | Not required |  |
| Data Protocol | TCP/IP, UDP, FTP/SFTP, push to NMHS server, email (Iridium) | TCP/IP, FTP, push to NMHS server | FTP, email, or manual download |  |
| Transmission Frequency | Real-time: ≤5-minute intervals during floods, ≤15 minutes normal; Event-based trigger for rapid rise | ≤15-minute intervals; Event-based trigger optional | Daily summary or on request |  |
| Tsunami Alert Transmission | Immediate transmission when rapid sea level change detected (>0.5m in 10 minutes) | Not required | Not required |  |
| Data Formats | SeaLevel format, NetCDF, CSV, JSON; Compatible with tsunami warning systems | CSV, JSON | CSV only |  |
| Data Plan | Minimum 3-year prepaid service included (for satellite) | Minimum 2-year prepaid service included | NMHS provides SIM or manual |  |
| Redundancy | Automatic failover between primary/secondary, store-and-forward during outages | Manual switchover if backup exists; store-and-forward | Store-and-forward only |  |
| Remote Monitoring | Full link quality monitoring, signal strength, data usage, battery voltage, alarms | Basic link monitoring | Not required |  |

Tsunami Warning Integration (Tier 1 Only):

System must be capable of sending alerts directly to:

☐ National Tsunami Warning Centre

☐ Regional Tsunami Service Provider

☐ PTWC (Pacific Tsunami Warning Center)

☐ SMS/email alerts to designated recipients

Data Flow Diagram Requirement regardless of tier:

The bidder must provide a detailed data flow diagram showing:

- Path from sensor → datalogger → transmission → NMHS server / tsunami warning centre

- All data formats at each stage

- Alert paths for tsunami and storm surge events

- Backup paths and failover mechanisms

- Store-and-forward operation during outages

- Points of manual intervention

Integration Testing:

The contractor shall demonstrate successful data transmission to:

☐ NMHS central server

☐ Tsunami warning centre

☐ Forecaster workstation display

  - Power System

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Primary Power Source | ☐ Solar (offshore) ☐ Solar + Wave hybrid ☐ Mains with UPS (coastal) | ☐ Solar ☐ Mains with battery | ☐ Solar ☐ Battery only |  |
| Solar Panel (Marine Grade) | Minimum 300W, marine-grade anodized frame, corrosion-resistant connectors, 25-year lifespan | Minimum 200W, marine-grade, corrosion-resistant | Minimum 100W, marine grade |  |
| Charge Controller | MPPT, 40A minimum, temperature compensation, remote monitoring, marine-grade coating | MPPT, 30A, marine-grade | PWM, 20A |  |
| Battery Bank | Deep-cycle AGM or Lithium (marine-grade), 300Ah minimum, 5-year design life, vented marine enclosure | Deep-cycle AGM, 200Ah minimum, 3-year design life | Deep-cycle AGM, 100Ah minimum |  |
| Autonomy | Minimum 21 days without sun (offshore), 14 days (coastal) | Minimum 14 days without sun | Minimum 7 days without sun |  |
| Power Distribution | Marine-grade distribution panel, circuit breakers, surge protection, galvanic isolation | Marine-grade fused distribution, surge protection | Fused distribution |  |
| Voltage Regulation | Regulated outputs: 12V, 5V, 24V if required; all sensors protected | Regulated 12V output | Battery voltage direct |  |
| Power Monitoring | Remote monitoring: voltage, current, battery health, charge status, alarms, remaining autonomy | Local display of voltage | Visual battery check |  |
| Enclosure | Marine-grade aluminium or stainless steel, IP66, waterproof cable glands, desiccant system, anti-condensation | Marine-grade, IP65, waterproof glands | Weatherproof, marine-grade |  |
| Connectors | All connectors marine-grade, waterproof when disconnected, locking types | Marine-grade, waterproof | Marine-grade |  |

Important Note for Marine Systems:

Marine environments are the most punishing for power systems. Requirements:

- Corrosion resistance: All components must have marine-grade coatings and materials

- Connector protection: All connectors must remain waterproof even when disconnected

- Galvanic isolation: Prevent electrolytic corrosion between dissimilar metals

- Storm survival: Solar panels and mounts must withstand cyclone-force winds

Sizing Calculation Requirement:

The bidder must provide detailed power budget showing:

- System total power consumption (watts) by component (including transmission)

- Daily energy requirement (watt-hours)

- Solar panel sizing calculation using worst-month marine solar insolation (including sea spray effects)

- Battery bank sizing calculation with depth-of-discharge

- Expected autonomy under worst-case conditions (cloudy periods, biofouling on solar panels)

  - Physical Infrastructure

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Tide Gauge Mounting | Dedicated gauge house or stilling well on wharf/pier, or bottom-mounted pressure sensor with secure cabling | Wharf/pier mounting or stilling well | Simple mounting to existing structure |  |
| Stilling Well (if used) | 300mm minimum diameter, marine-grade materials, perforated intake, accessible for cleaning, anti-biofouling measures | 200mm diameter, accessible | Not required |  |
| Wave Buoy Hull | 0.9-1.5m diameter, marine-grade polyurethane or foam-filled, high-visibility yellow, radar reflector, navigation light, mooring attachment points | 0.7-1.2m diameter, marine-grade, high-visibility | 0.5-0.9m diameter, basic marine buoy |  |
| Buoy Mooring System | Dual redundant mooring, galvanized or stainless steel chain, swivels, shackles, concrete or clump weight anchor, backup mooring line | Single mooring with appropriate scope, galvanized chain, anchor | Basic mooring for sheltered waters |  |
| Navigation Aids | LED navigation light (2-5 nautical mile visibility), radar reflector, top-mark as per IALA requirements | Navigation light, radar reflector | Navigation light where required |  |
| Anti-biofouling | Copper-based anti-fouling paint on hull, ultrasonic anti-fouling for sensors (optional), wipers for optical sensors | Anti-fouling paint, sensor cleaning schedule | Anti-fouling paint |  |
| Lightning Protection | Comprehensive lightning protection for all above-water components, surge arrestors on all lines | Lightning rod, surge arrestors | Basic protection |  |
| Grounding (Coastal Sites) | <10 ohm ground resistance, separate ground for power and signal, cathodic protection if needed | <25 ohm ground resistance | Basic grounding |  |
| Cabling (Submarine) | Double-armored submarine cable, water-blocked, termination chambers at both ends, cable markers | Armored cable, protected | Standard marine cable |  |
| Cabling (Above Water) | UV-resistant, marine-grade, all cables in conduit, waterproof glands, labeled both ends | Marine-grade, protected | Outdoor marine cable |  |
| Security | Anti-theft bolts, locking enclosures, security marking, GPS tracking for buoys (optional) | Locking enclosures, anti-theft measures | Padlocks |  |

Site Preparation Requirements:

The contractor shall provide detailed site preparation guide including:

- For coastal sites: Wharf/pier attachment specifications, stilling well installation, cable routing, grounding

- For offshore buoys: Deployment procedure, mooring design calculations, navigation authority approvals

- For bottom-mounted sensors: Deployment/recovery procedure, diver requirements if applicable

- Permits required: Navigation authority, coastal zone management, environmental permits, customs for international waters

  - Software & Data Management

| Component | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Bidder Response |
| --- | --- | --- | --- | --- |
| Logger Configuration Software | Full-featured, graphical interface, offline programming, data retrieval, real-time monitoring, wave processing configuration | Full-featured, offline programming | Basic configuration tool |  |
| Data Viewing Software | Real-time and historical data display, tide prediction, storm surge display, wave spectra, SST trends | Real-time display, basic tide graphs | Not required |  |
| Tsunami Detection Software | Real-time sea level anomaly detection, configurable thresholds, automatic alert generation | Not required | Not required |  |
| Data Management System | Compatible with national oceanographic database, GLOSS data archive, automatic data push to international centres (UHSLC, WMO) | Compatible with national database, data push | CSV export for manual import |  |
| SeaLevel Format Compliance | Full SeaLevel format output for GLOSS data exchange | SeaLevel format preferred | Not required |  |
| NetCDF Compliance | NetCDF output for climate and research applications | Optional | Not required |  |
| Web Interface | Optional: web-based data viewing for public, port users, fishermen | Optional | Not required |  |
| Mobile Application | Optional: mobile app for data viewing and alerts | Optional | Not required |  |
| License Model | Perpetual license, no recurring fees | Perpetual license | Included |  |
| Software Updates | 5 years free updates included | 3 years updates included | As available |  |
| API Access | Full open API for integration with other systems | Documented data access methods | Not required |  |

  - Integration with Existing Systems

The proposed system must integrate with the following systems:

| System | Integration Requirement | Compliant? |
| --- | --- | --- |
| [e.g., National Tsunami Warning Centre] | Real-time data feed with tsunami alert messages |  |
| [e.g., GLOSS Data Archive] | SeaLevel format data push, daily |  |
| [e.g., WMO GTS/WIS 2.0] | BUFR generation for marine data |  |
| [e.g., Port Operations System] | Real-time tide and wave data |  |
| [e.g., National Oceanographic Database] | Automatic data push |  |
| [e.g., CliDE] | Compatible data formats |  |

Integration Testing:

The contractor shall demonstrate successful integration during:

- Factory Acceptance Test (simulated environment)

- Site Acceptance Test (live at installation)

- Tsunami Alert Test (simulated event) for Tier 1

## Technical Specification by Tier

The specifications in this table are derived from the WRP Guidance Document: Standardisation, Optimisation and Maintenance of Earth Observation Equipment in the Pacific Islands, Section 5.2.3 (Tier Sensor Specification by Variable – Marine). Refer to the Guide for detailed sensor accuracy and uncertainty data.

  - Sea Level (Tide) Sensor Specifications

Tier 1 - Reference Station (GLOSS Compliant)

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Measurement Range | 0-15m (or site-specific tidal range + storm surge allowance) |  |  |
| Accuracy | ±0.5 cm (radar), ±0.1% of full scale (pressure) or ±1 cm, whichever is greater |  |  |
| Resolution | 1 mm |  |  |
| Long-term Stability | <1 cm per decade for climate applications |  |  |
| Sampling Rate | 1-minute averages, 1-second for tsunami detection |  |  |
| Tsunami Mode | 15-second or better sampling when triggered |  |  |
| Output | SDI-12, RS-232, or 4-20mA |  |  |
| Redundancy | Dual sensors (different technologies recommended: e.g., radar + pressure) |  |  |
| Materials | Marine-grade materials (316 stainless steel, titanium, marine aluminium) |  |  |
| Ingress Protection | IP68 for underwater components, IP66 for above-water |  |  |

Special Requirements for Tier 1:

- Dual sensor redundancy recommended for critical sites

- Factory calibration certificate traceable to ISO 17025

- Annual calibration verification

- Real-time datum control (continuous GPS connection recommended)

- Co-located with CGPS for vertical land movement monitoring (for climate sites)

Tier 2 - Synoptic Station

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Measurement Range | 0-10m minimum (or site-specific) |  |  |
| Accuracy | ±1 cm |  |  |
| Resolution | 1 cm |  |  |
| Sampling Rate | 1-minute averages |  |  |
| Output | SDI-12 or 4-20mA |  |  |
| Materials | Marine-grade materials |  |  |
| Ingress Protection | IP68 for underwater, IP65 for above-water |  |  |

Tier 3 - Basic Station

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Measurement Range | 0-5m minimum |  |  |
| Accuracy | ±2-5 cm |  |  |
| Resolution | 1 cm |  |  |
| Sampling Rate | 5–15-minute averages |  |  |
| Output | SDI-12 or analog |  |  |
| Ingress Protection | IP67 |  |  |

  - Wave Sensor Specification (Buoy-Mounted)

Tier 1 – Reference Wave Buoy

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Wave Height Range | 0-20m |  |  |
| Wave Height Accuracy | ±0.2m or ±2% |  |  |
| Wave Period Range | 1-30 seconds |  |  |
| Wave Period Accuracy | ±0.5 seconds |  |  |
| Wave Direction Range | 0-360° |  |  |
| Wave Direction Accuracy | ±5° |  |  |
| Sampling Rate | 1-2 Hz minimum |  |  |
| Burst Duration | 20-30 minutes every 1-3 hours, configurable |  |  |
| Output Parameters | Hs, Hmax, Tz, Tp, Tmean, Direction (peak and mean), Spread, Wave spectra |  |  |
| Sensor Type | Accelerometer + compass, or GPS-based |  |  |
| Data Processing | Onboard wave spectra calculation |  |  |

Tier 2 – Synoptic Wave Buoy

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Wave Height Range | 0-15m |  |  |
| Wave Height Accuracy | ±0.3m or ±5% |  |  |
| Wave Period Accuracy | ±1.0 seconds |  |  |
| Wave Direction Accuracy | ±10° |  |  |
| Sampling Rate | 1 Hz |  |  |
| Output Parameters | Hs, Tz, Tp, Direction |  |  |

Tier 3 – Basic Wave Sensor

| Parameter | Specification | Compliance (Y/N) | Evidence |
| --- | --- | --- | --- |
| Wave Height | ±0.5m or ±10% |  |  |
| Wave Period | ±2 seconds |  |  |
| Direction | ±20° (if available) |  |  |
| Output | Basic wave statistics |  |  |

  - Sea Surface Temperature (SST) Specifications

| Parameter | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Compliance (Y/N) | Included? |
| --- | --- | --- | --- | --- | --- |
| Measurement Range | -5°C to +40°C | -5°C to +40°C | -5°C to +40°C |  | ☐ |
| Accuracy | ±0.1°C | ±0.3°C | ±0.5°C |  | ☐ |
| Resolution | 0.01°C | 0.1°C | 0.1°C |  | ☐ |
| Response Time | <30 seconds | <1 minute | <2 minutes |  | ☐ |
| Long-term Stability | <0.05°C per year | <0.1°C per year | <0.2°C per year |  | ☐ |
| Sensor Type | Platinum RTD or high-quality thermistor | Thermistor | Thermistor |  | ☐ |
| Depth | 1-3m below surface (or sea chest for coastal sites) | 1-3m | Surface |  | ☐ |

  - Barometric Pressure (Marine) Specifications

| Parameter | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Compliance (Y/N) | Included? |
| --- | --- | --- | --- | --- | --- |
| Accuracy | ±0.2 hPa | ±0.5 hPa | ±1.0 hPa |  | ☐ |
| Resolution | 0.1 hPa | 0.1 hPa | 0.5 hPa |  | ☐ |
| Range | 800-1100 hPa | 800-1100 hPa | 800-1100 hPa |  | ☐ |
| Temperature Compensation | Full | Full | Basic |  | ☐ |

  - Conductivity / Salinity (Optional)

| Parameter | Tier 1 Specification | Tier 2 Specification | Tier 3 Specification | Compliance (Y/N) | Included? |
| --- | --- | --- | --- | --- | --- |
| Conductivity Range | 0-70 mS/cm | 0-70 mS/cm | 0-70 mS/cm |  | ☐ |
| Conductivity Accuracy | ±0.01 mS/cm | ±0.1 mS/cm | ±0.5 mS/cm |  | ☐ |
| Salinity Accuracy | ±0.1 PSU | ±0.5 PSU | ±1.0 PSU |  | ☐ |
| Temperature Compensation | Automatic | Automatic | Basic |  | ☐ |
| Biofouling Protection | Anti-fouling wipers or copper shielding required | Wipers recommended | Manual cleaning |  |  |

## Spare Parts Requirements by Tier

#### Tier 1 Spare Parts Kit (per system)

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Sensors | Sea level sensor (complete) - primary type | 1 |  |  |
|  | Sea level sensor (secondary/redundant type) | 1 |  |  |
|  | SST sensor | 1 |  |  |
|  | Barometric pressure sensor | 1 |  |  |
|  | Wave sensor (complete) | 1 |  |  |
| Data Acquisition | Data logger (complete, programmed) | 1 |  |  |
|  | Communication modem (same as installed) | 1 |  |  |
|  | GPS receiver | 1 |  |  |
| Power System | Solar panel (same rating, marine grade) | 1 |  |  |
|  | Charge controller | 1 |  |  |
|  | Battery (marine deep-cycle, same specification) | 2 |  |  |
| Mooring/Infrastructure | Complete mooring chain set (for buoy) | 1 set |  |  |
|  | Shackles, swivels, connectors | 2 sets |  |  |
|  | Anchor (if practical) | 1 |  |  |
|  | Navigation light | 1 |  |  |
| Cables | Full cable set (sensor, power, telemetry) | 2 sets |  |  |
|  | Waterproof connectors | 2 sets |  |  |
| Consumables | Desiccant packs (marine-grade) | 20 |  |  |
|  | Anti-fouling paint (if applicable) | 1 kit |  |  |
|  | Cable ties (UV-stabilized, marine-grade) | 100 |  |  |
|  | Lubricant for marine connectors | 2 tubes |  |  |
| Tools | Field verification/calibration kit (portable pressure reference, temperature reference) | 1 |  |  |
|  | Connector maintenance kit | 1 |  |  |
|  | Diving tools (if applicable) | As required |  |  |
| Safety | Buoy retrieval equipment (grappling hook, line) | 1 set |  |  |
|  | Personal safety equipment for deployment | As required |  |  |
| TOTAL (Tier 1) |  |  |  |  |

#### Tier 2 Spare Parts Kit (per system)

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Sensors | Sea level sensor | 1 |  |  |
|  | Barometric pressure sensor | 1 |  |  |
|  | SST sensor (if installed) | 1 |  |  |
| Data Acquisition | Data logger | 1 |  |  |
|  | Communication modem | 1 |  |  |
| Power System | Battery | 2 |  |  |
|  | Solar panel | 1 |  |  |
| Mooring/Infrastructure | Critical mooring components | 1 set |  |  |
| Cables | Critical cables | 1 set |  |  |
| TOTAL (Tier 2) |  |  |  |  |

#### Tier 3 Spare Parts Kit (per 5 systems recommended)

| Component Category | Item | Minimum Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Sensors | Sea level sensor | 2 |  |  |
|  | SST sensor (if installed) | 1 |  |  |
| Power | Battery | 4 |  |  |
| Mooring | Basic repair kit | 1 |  |  |
| Tools | Basic toolkit | 1 |  |  |
| TOTAL (Tier 3) |  |  |  |  |

## Maintenance Schedule

Tier 1 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Daily (remote) | Remote data check, verify transmissions, review data quality, check battery voltage, check for alarms | 30 min | Oceanographer |
| Weekly (remote) | Review wave statistics, check for sensor drift, verify tsunami detection system | 1 hour | Specialist |
| Quarterly (site) | Coastal sites: Visual inspection, clean sensors, check cables, verify against manual reading, battery check, solar panel cleaning | 2-4 hours | Technician |
| Bi-annually (buoy) | Buoy retrieval (if possible) OR diver inspection: Check hull integrity, clean sensors, anti-fouling inspection, mooring inspection, replace desiccant | 1 day | Specialist + Diver |
| Annually | Full system calibration verification, battery capacity test, software updates, mooring replacement if needed | 2-3 days | Specialist |
| Post-Storm Event | Site/buoy inspection after major cyclone/storm, check for damage, sensor positioning, mooring condition | As needed | Technician |
| 5-Year | Major refurbishment: replace all sensors, replace batteries, replace critical cables, full system recertification, repaint hull if needed | 1 week | Manufacturer |

Tier 2 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Weekly (remote) | Remote data check | 30 min | Technician |
| Quarterly | Coastal sites: Site visit, visual inspection, sensor cleaning, battery check | 2 hours | Technician |
| Annually | Full site inspection, sensor verification, battery replacement if needed | 4 hours | Technician |

Tier 3 Maintenance

| Frequency | Tasks | Estimated Time | By Whom |
| --- | --- | --- | --- |
| Monthly (remote) | Remote data check | 30 min | Technician |
| Bi-annually | Site visit: Visual inspection, clean sensor, battery check | 1-2 hours | Technician or Observer |
| Annually | Battery replacement, full functional check | 2-3 hours | Technician |

## Field Verification & Calibration

This section defines requirements for two distinct activities. Field Verification is the on-site check of instrument performance against a portable reference. Calibration is a formal, traceable comparison against a national or international standard, typically performed in a laboratory setting. See the WRP Guidance Document: Standardisation, Optimisation and Maintenance of Earth Observation Equipment in the Pacific Islands, Section 7.4 for more detail.

  - Field Verification Equipment

The contractor shall provide field verification equipment:

| Item | Tier 1 | Tier 2 | Tier 3 | Included? |
| --- | --- | --- | --- | --- |
| Portable Water Level Meter | Accuracy ±2mm, 30m tape, with buzzer/light, marine grade | Accuracy ±5mm, 20m tape | Not required | ☐ |
| Barometric Reference | Accuracy ±0.2 hPa, calibrated, marine grade | Accuracy ±0.5 hPa | Not required | ☐ |
| Temperature Reference | Accuracy ±0.1°C, NIST traceable, waterproof | Accuracy ±0.3°C | Not required | ☐ |
| Conductivity Standard | If salinity sensors installed | If salinity sensors installed | Not required | ☐ |
| Diver Inspection Tools | Camera, cleaning tools, spare parts | Not required | Not required | ☐ |
| GPS Survey Equipment | For datum verification (Tier 1 climate sites) | Not required | Not required | ☐ |

  - Calibration Requirements

| Component | Tier 1 | Tier 2 | Tier 3 | Included? |
| --- | --- | --- | --- | --- |
| Sea Level Sensor | Annual factory or laboratory calibration | 2-year calibration | As failed replacement | ☐ |
| SST Sensor | Annual laboratory calibration | 2-year calibration | As failed | ☐ |
| Barometric Pressure | Annual laboratory calibration | 2-year calibration | As failed | ☐ |
| Wave Sensor | 2-year factory calibration | 3-year verification | As failed | ☐ |
| Conductivity/Salinity | Annual calibration with standards | 2-year calibration | As failed |  |
| Field Verification Equipment | Annual calibration at accredited lab | Annual calibration | N/A |  |
| GPS/GNSS (for datum) | Annual verification | Not required | N/A |  |

## Warranty & Services

  - Warranty Requirements

| Component | Tier 1 | Tier 2 | Tier 3 | Bidder Offer |
| --- | --- | --- | --- | --- |
| Warranty Period | Minimum 3 years comprehensive | Minimum 2 years comprehensive | Minimum 1 year comprehensive |  |
| Coverage | Parts, labour, shipping both ways | Parts, labour, shipping to vendor | Parts only, return to vendor |  |
| Marine-Specific Coverage | Corrosion, biofouling damage, storm damage (optional) | Corrosion coverage | Basic only |  |
| Response Time | <24 hours for critical failure acknowledgment | <48 hours for critical failure | <5 working days |  |
| Remote Support | 24/7 hotline, remote diagnostics included | Business hours hotline, remote diagnostics | Email support |  |
| On-site Support | Guaranteed within 14 days for critical failures (weather permitting) | Within 30 days if remote cannot resolve | Not included |  |

  - Tsunami / Storm Surge Support (Tier 1)

| Requirement | Specification | Bidder Offer |
| --- | --- | --- |
| Priority Support | Critical failures always prioritised (24/7/365) |  |
| Rapid Deployment | Temporary replacement sensor available within 1 week |  |
| Extended Warranty | Storm damage coverage (optional) |  |

  - Service Level Agreement (SLA) Requirements

The contractor shall provide a detailed SLA including:

| SLA Element | Tier 1 Requirement | Tier 2 Requirement | Bidder Offer |
| --- | --- | --- | --- |
| Data availability target | 99% (excluding scheduled maintenance) | 95% |  |
| Scheduled maintenance | Maximum 4 days per year, 4 weeks’ notice | Maximum 5 days per year, 2 weeks’ notice |  |
| Helpdesk hours | 24/7/365 | Business hours with after-hours emergency contact |  |
| Critical failure response | <2 hours acknowledgment, <24 hours remote attempt | <24 hours acknowledgment |  |
| Spare parts delivery | <1 week to regional hub, <2 weeks to site | <2 weeks to site |  |

  - Post-Warranty Support Options

The bidder must provide pricing for post-warranty support:

| Option | Description | Annual Cost | Notes |
| --- | --- | --- | --- |
| Option 1: Comprehensive | Full coverage as during warranty |  |  |
| Option 2: Parts only | Spare parts at fixed prices |  |  |
| Option 3: Time & materials | Support charged as needed |  |  |
| Option 4: Training refresh | Refresher training for new staff |  |  |
| Option 5: Mooring replacement | Scheduled mooring replacement service |  |  |

## Training Requirements

| Training Type | Audience | Duration | When | Must Include |
| --- | --- | --- | --- | --- |
| Oceanographer training | Marine forecasters, tsunami warning staff | 2-3 days | Before installation | Data interpretation, tsunami detection, storm surge, tide predictions, quality control |
| Technician training | Field technicians | 5 days | During installation | Sensor installation, maintenance, troubleshooting, field verification, safety at sea |
| Deployment training | Marine operations staff | 3-5 days | Before deployment | Buoy deployment/recovery procedures, mooring techniques, navigation safety |
| Diver training | Diver-qualified technicians | 2 days | During installation | Underwater sensor maintenance, inspection procedures, safety |
| Software training | Data managers | 2-3 days | After installation | Database integration, data export, tsunami alert configuration |
| Community observer training | Local observers (Tier 3) | 1 day | At installation | Basic checks, manual readings, reporting issues |

Training materials required:

☐ User manuals (printed + digital) in English

☐ Maintenance manuals with exploded diagrams (waterproof for field use)

☐ Quick reference guides (laminated for field use) – 2 copies per station

☐ Video tutorials (recommended)

☐ Spare parts catalog with part numbers and photos

☐ Deployment/recovery procedures (with diagrams)

☐ Troubleshooting flowchart

☐ Preventative maintenance checklist (laminated)

☐ Safety manual (working at sea, small boat safety, driving if applicable)

☐ Tsunami alert test procedures (Tier 1)

Training Evaluation:

- Pre- and post-training assessments required

- Practical demonstration of competency (each participant must perform tasks)

- Certificate of completion for each participant

- Training report with recommendations for additional support

## Technical Documentation

The contractor must provide comprehensive technical documentation:

  - System Documentation

| Document | Description | Format | Quantity |
| --- | --- | --- | --- |
| System Design Document | Overall architecture, sensor specifications, wiring diagrams, telemetry configuration | Digital + 2 hard copies | 2 sets |
| Installation Manual | Site preparation, sensor installation, stilling well construction (coastal), buoy deployment procedures, mooring specifications | Digital + 2 hard copies | 2 sets |
| User Manual | Daily operation, data viewing, basic troubleshooting | Digital + 2 hard copies | 3 sets |
| Maintenance Manual | Preventive maintenance schedules, troubleshooting, repairs, sensor cleaning procedures | Digital + 2 hard copies | 3 sets |
| Technical Drawings | Mechanical drawings, electrical schematics, stilling well details, buoy hull drawings, mooring diagrams | Digital (PDF + CAD) + 2 hard copies | 2 sets |
| Parts Catalog | Complete parts list with part numbers, suppliers, cross-reference | Digital + 2 hard copies | 3 sets |
| Software Documentation | User guides, API documentation, data formats (SeaLevel, NetCDF), alert configuration | Digital | 3 sets |
| Calibration Procedures | Step-by-step calibration, traceability information, field verification methods | Digital + 2 hard copies | 3 sets |
| Safety Manual | Working at sea, small boat safety, diving procedures (if applicable), electrical safety | Digital + 2 hard copies | 3 sets |
| Mooring Manual | Mooring design calculations, deployment procedures, inspection schedule, replacement criteria | Digital + 2 hard copies | 2 sets |
| As-Built Documentation | Final as-installed drawings, GPS coordinates, deployment photos, configuration settings, datum information | Digital + 2 hard copies | 2 sets |

  - Documentation Quality Requirements

- All documentation must be in English

- Language must be clear, avoiding unnecessary jargon

- Diagrams and illustrations must be used extensively

- Troubleshooting guides must include decision trees

- All documents must include revision control and date

- Hard copies must be durable (weather-resistant covers for field manuals)

- Digital copies must be searchable PDFs

- Critical procedures must have bilingual summaries (English + local language) where possible

  - Site-Specific Documentation

For each installed station, the contractor shall provide:

- Station location map and coordinates (WGS84)

- Site photographs (4 directions, close-ups of each component)

- Sensor heights and orientations

- Configuration file for datalogger

- Network and communication settings

- Contact list for support

## Lifecycle Cost Calculation

| Cost Category |  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 | 5-Year Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Capital costs | Sensors (level, wave, SST, etc.) |  |  |  |  |  |  |
|  | Datalogger |  |  |  |  |  |  |
|  | Telemetry equipment |  |  |  |  |  |  |
|  | Power system (solar, batteries) |  |  |  |  |  |  |
|  | Buoy hull / coastal infrastructure |  |  |  |  |  |  |
|  | Mooring system |  |  |  |  |  |  |
|  | Navigation aids |  |  |  |  |  |  |
|  | Installation / deployment |  |  |  |  |  |  |
|  | Training |  |  |  |  |  |  |
| Recurring costs | Data plans (satellite/cellular) |  |  |  |  |  |  |
|  | Consumables (desiccant, etc.) |  |  |  |  |  |  |
|  | Anti-fouling paint |  |  |  |  |  |  |
|  | Batteries |  |  |  |  |  |  |
| Maintenance | Spare parts |  |  |  |  |  |  |
|  | Calibration services |  |  |  |  |  |  |
|  | Software support / licenses |  |  |  |  |  |  |
|  | Support contract |  |  |  |  |  |  |
|  | Diver services (if applicable) |  |  |  |  |  |  |
|  | Vessel charter for maintenance |  |  |  |  |  |  |
|  | Mooring replacement (every 2-3 years) |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |

Note: Marine systems have significantly higher operational costs than land-based systems due to vessel charters, diver services, and mooring replacements. Ensure these are realistically budgeted.

- Include all costs in [USD or local currency]

- Assume normal operation, no catastrophic failures

- Include inflation if multi-year contract

- Separate optional items clearly

## Pacific Requirements

All marine observation systems must meet these additional requirements:

| Requirement | Specification | Compliant? |
| --- | --- | --- |
| Corrosion protection | Marine-grade materials (316 stainless steel, titanium, marine aluminium, copper alloys), sacrificial anodes, marine-grade coatings |  |
| UV resistance | All exposed components UV-stabilized, minimum 5-year UV life in tropical conditions |  |
| Cyclone resistance | Buoy and sensors rated for Category 5 cyclone conditions (250 km/h winds, 15m+ waves); coastal structures rated for storm surge |  |
| Salt spray protection | All components sealed to IP66 minimum (above water), IP68 (submerged), marine-grade connectors |  |
| Biofouling resistance | Anti-fouling paint on hull, copper shielding or wipers on sensors, regular cleaning schedule |  |
| Temperature range | Operational: 0°C to +45°C (tropical waters can exceed 35°C) |  |
| Corrosion in tropics | Accelerated corrosion in warm, saline waters requires higher-grade materials |  |
| Lightning protection | Comprehensive protection; Pacific has highest lightning density globally |  |
| Power autonomy | 21 days minimum for offshore buoys (cyclones bring extended cloud cover) |  |
| Navigation safety | IALA-compliant navigation lights, radar reflectors, high-visibility colours |  |
| Theft/vandalism prevention | Anti-theft bolts, locking enclosures, remote monitoring, GPS tracking for buoys |  |
| Shark/animal damage | Protected cabling, robust housings, avoidance of attractants |  |
| Tsunami readiness | Real-time data with redundant telemetry, immediate alerting capability |  |
| Customs clearances | Equipment may cross international waters; import/export permits for servicing |  |
| Local support | Design for maintenance by local technicians with limited diving/marine resources |  |

## Compliance Checklist

Before finalising your tender, verify:

| Check | Completed? |
| --- | --- |
| Tier selection matches your operational needs | ☐ |
| All required variables selected | ☐ |
| Water depth and location data provided for all sites | ☐ |
| System type clearly defined (tide gauge, wave buoy, etc.) | ☐ |
| System architecture fully defined | ☐ |
| Telemetry appropriate for site (cellular/satellite) | ☐ |
| Sea level sensor type appropriate for site conditions | ☐ |
| Power system sized for location (21-day autonomy for offshore) | ☐ |
| Biofouling countermeasures specified | ☐ |
| Wave sensor specifications match expected sea state | ☐ |
| Mooring designed for site depth and conditions | ☐ |
| Navigation aids included where required | ☐ |
| All sensor specifications are included and match the tier selected | ☐ |
| Data transmission requirements specified and appropriate for the site | ☐ |
| Physical infrastructure requirements clear and appropriate for site conditions | ☐ |
| Software and integration requirements specified | ☐ |
| Spare parts kit matches tier requirement and network size | ☐ |
| Maintenance schedule defined and resourced | ☐ |
| Field verification equipment included |  |
| Warranty and SLA requirements specified | ☐ |
| Training requirements fully defined and specified | ☐ |
| Technical documentation requirements included | ☐ |
| Pacific requirements included | ☐ |
| Lifecycle cost worksheet completed | ☐ |
| Budget confirmed for 5-year operation | ☐ |
| Site surveys completed for all locations | ☐ |

## Additional Information for Bidders

| Site | Type | Latitude | Longitude | Water Depth (m) | Distance Offshore (km) | Access | Max Wave Height (m) | Tidal Range (m) | Navigation Hazard | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |

  - Site Information (to be completed by NMHS)

  - Existing Equipment at Sites

| Site | Existing Logger | Existing Sensors | Compatible? |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

  - Local Support Resources Available

☐ Vessel available (size: _____ m)

☐ Divers available (certified: ☐ Yes ☐ No)

☐ Local marine workshop

☐ Port facilities

☐ Navigation authority contact

  - Special Local Requirements

Import duties: ☐ NMHS responsible ☐ Bidder responsible

Customs clearance: ☐ NMHS assists ☐ Bidder responsible

Navigation permits: ☐ Required ☐ Not required

Environmental permits: ☐ Required ☐ Not required

Traditional fishing grounds / cultural sites: ☐ Yes (please specify: _______) ☐ No

Local taxes: ☐ Applicable ☐ Exempt (specify)

Work permits: ☐ Required ☐ Not required

Site access restrictions: _________________________________